SHDOMPP - Spherical Harmonic Discrete Ordinate Method

for Plane-Parallel Atmospheric Radiative Transfer

OVERVIEW

SHDOMPP computes unpolarized radiative transfer in a plane-parallel

medium for either collimated solar and/or thermal emission sources of

radiation.   Currently Lambertian and RPV land surface BRDF surface

reflection are supported.  The atmosphere optical properties are

specified with an input file listing the layer interface heights and

temperatures, and layer optical depth, single scattering albedo, and

Legendre series coefficients of the phase function.  Two output files

are generated: one with the upwelling and downwelling hemispheric fluxes

and actinic fluxes at the layer boundaries and another with radiances at

specified heights and directions.  The SHDOMPP calculation may be either

monochromatic or spectrally integrated with a k-distribution.

The SOLVE_SHDOMPP subroutine in shdompp.f90 is designed to be simple to

call in user programs, and its inputs and outputs are documented in the

code.  SHDOMPP is written in Fortran 90.

SIMILARITIES AND DIFFERENCES FROM SHDOM

The algorithm is based on that of SHDOM, having iterations between

calculating the source function in spherical harmonics and integrating

the radiative transfer equation in discrete ordinates.  A major

difference from SHDOM is that the medium is represented by layers which

have uniform optical properties, like DISORT, rather than linear

interpolation of the optical properties between grid points.  Another

difference from SHDOM is that the source function is interpolated within

each layer by a cubic polynomial (quadratic at the layer boundaries).

This gives higher accuracy in the source function integration, allowing

fewer points (levels) to be used for the same accuracy.  Like SHDOM, an

adaptive grid is used, but the criterion for deciding to split sublayers

is different.  The current SHDOMPP cell splitting criterion is the rms

radiance difference between using the cubic/quadratic and linear source

function interpolation in the RTE integration.  Thus it is a measure of

how much the curvature of the source function affects the radiances. The

splitting criterion is normalized, so it is a relative measure, whereas

the criterion in SHDOM is an absolute measure.   Unlike in SHDOM the

spherical harmonic series of the source function is not adaptive.  The

delta-M method is always turned on internally, and is not a user

specified parameter.  SHDOMPP is written completely in Fortran 90 and

exists in one file, while SHDOM is mainly Fortran 77.

INPUT PARAMETERS

The parameters are input in the routine USER_INPUT.  USER_INPUT uses

READ statements from stdin in response to written messages.  The

USER_INPUT routine echos the input values so that a useful "log" file

output is produced when run non-interactively (see scripts).

Parameter      Description

PROPFILE       name of the input medium property file

CKDFILE        name of the input correlated k-distribution file (or NONE)

The CKD file has the same format as for SHDOM.

NMU, NPHI      number of discrete ordinates covering -1<mu<1 and 0<phi<2pi

SRCTYPE        S' for solar source, 'T' for thermal source, 'B' for both

Following for Solar or Both source

SOLARFLUX      top of medium solar flux on a horizontal surface (any units)

SOLARMU        cosine of the solar zenith angle (this represents the

direction of travel of the solar beam, so is forced

to be negative although it can be specified positive).

SKYRAD         isotropic diffuse radiance incident from above

Following for Thermal or Both source

SFCTEMP        temperature of surface (Kelvin)

Following for Thermal source

SKYRAD         blackbody temperature (Kelvin) of radiation incident from above

SFCTYPE        type of surface: L for Lambertian, R for RPV BRDF

SFCPARMS(*)    array of surface parameters:

SFCPARM(1) is albedo for SFCTYPE='L',

SFCPARM(1:3) are rho0, k, Theta for RPV.

(Rahman, Pinty, Verstraete, 1993: J. Geophys. Res., 98, 20791)

WAVELEN        wavelength in microns (if <=0 then wavenumbers used instead)

WAVENO(1:2)    wavenumber range (cm^-1) (WAVENO(1)<WAVENO(2))

SPLITACC       cell splitting accuracy; sublayers that have the adaptive

splitting criterion above this value are split.

It is relative to the typical radiance level.

Set to zero or negative for no adaptive cell splitting.

SOLACC         solution accuracy - tolerance for solution criterion

The solution criterion is the rms change in the source

function in one iteration, so the true solution accuracy

is considerably larger when convergence is slow.

ACCELFLAG      T (true) to do the sequence acceleration. An acceleration

extrapolation of the source function is done every other

iteration.

MAXITER        maximum number of iterations allowed

FLUXFILE       name of output file for fluxes

FLUXPARM       1 for output at the input layer boundaries, 2 for output at

the adaptive sublayer boundaries (except for k-distribution)

RADFILE        name of output file for radiances.  The radiances are output

at NMUOUT*NPHIOUT directions for each of NZOUT heights.

NZOUT          number of height levels for output radiance

ZOUT           array of height levels for output radiance

NMUOUT         number of cosine zenith angles for output radiance

MUOUT          array of cosine zenith angles for output radiance

NPHIOUT        number of azimuth angles for output radiance

PHIOUT         array of azimuth angles for output radiance (degrees)

There are also three parameters set in the code at the top of shdompp.f90:

MAXDELTAU=1.0         maximum layer optical depth allowed in initial sublayers

SPLITTING_FACTOR=10.0 max number of internal grid points is this factor

times the number of input layers + optical depth

(We have to guess the maximum array size for the adaptive grid.)

PRNT=.TRUE.           true for interation printing, false for no printing

There is a parameter set at the top of the SOLVE_SHDOMPP subroutine:

MINDELTAUSPLIT=1.0E-3  minimum scaled optical depth for which an input

layer is divided into two sublayers or

adaptively split

PROPERTY FILE FORMAT

The medium optical properties are specified for uniform layers from the

top of the atmosphere down to the surface.  The file is plain ascii and

read by Fortran free format (so most "lines" may be broken across

lines).  The property file has a header of two lines followed by one

line for each layer:

Nlay

Ztoa  Ttoa

Zbot  Tbot  tau  ssalb  Nleg  Chi1 Chi2 ...

. . .

"Nlay" is the number of layers.  "Ztoa" is the height and "Ttoa" is the

temperature (K) of the top boundary of the first layer.  "Zbot" is the

height and "Tbot" is the temperature (K) of the bottom boundary of the

layer.  "tau" is the layer optical thickness (not the cumulative optical

depth), and "ssalb" is the single scattering albedo of the layer.

"Nleg" is the order of the Legendre series for the phase function, i.e.

it is the number of Legendre coefficients that follow.  "Chi?" are the

Legendre series phase function coefficients, starting with l=1 (so that

Chi1 is 3 times the asymmetry parameter).  The heights must decrease,

but can have any units, except they must match the k-distribution file

(usually units of km) when input.

For solar reflection problems the whole Legendre phase function series

is used to calculate output radiances using the Nakajima and Tanaka

(1988) TMS method.  Therefore, the full Legendre series (until ChiN<<1)

should be input.

USING A K-DISTRIBUTION

Spectral integration across a wavenumber band may be performed using a

k-distribution.  If the CKDFILE input is NONE then a monochromatic

calculation is performed, otherwise the CKDFILE contains the molecular

absorption coefficients of the k-distribution for a particular

atmospheric profile.  The format of the CKD file is the same as that for

SHDOM.  SHDOMPP loops over the k-distribution calling the monochromatic

solver (SOLVE_SHDOMPP) once for each k.  For each k the molecular

absorption profile is integrated over each layer in the input property

file (assuming linear interpolation between the levels in the CKD file),

and added to the optical properties specified in the property file.

ACCURACY ISSUES

The accuracy of SHDOMPP output radiances and fluxes is determined by the

angular resolution, chosen by NMU and NPHI, the spatial resolution,

chosen by SPLITACC (and the input property file and the MAXDELTAU

parameter), and the solution convergence, chosen by SOLACC.  By contrast

the accuracy of DISORT is determined solely by the number of streams

(NMU).  A good choice for solar reflection problems is NPHI=2*NMU, but

for high Sun or low viewing zenith angles or a more isotropic phase

function a smaller NPHI could be accurate.  For thermal emission

problems set NPHI=1.  Note: NPHI is the number of azimuth angles in 0 to

2*pi, but only the range 0 to pi is actually used internally.   There is

seldom any point in having the SPLITACC and SOLACC parameters set very

small if the accuracy is limited by low angular resolution (small NMU).

How the actual accuracy depends on the input parameters should be

investigated with a convergence test for problems similiar to the ones

you want to carry out.  See the "run_compare_shdompp" script for

examples of convergence testing.  See "compare_shdompp_disort.txt" for

the accuracy and timing results comparing SHDOMPP and DISORT for a

variety of atmospheric radiative transfer problems.

RUNNING SHDOMPP

First compile shdompp.f90, and the other programs in the distribution if

desired.  Edit the makefile to specify your Fortran 90 compiler and

flags.  The distribution contains two scripts to illustrate how to run

SHDOMPP:

run_shdompp_examples   five radiative transfer examples, including a

k-distribution test.

run_compare_shdompp    extensive accuracy and timing comparison of

SHDOMPP and DISORTSH for monochromatic RT.

OTHER PROGRAMS in DISTRIBUTION

DISORTSH

disortsh.f90 is a driver program for DISORT version 2.1, which is

available via anonymous ftp from

ftp://climate.gsfc.nasa.gov/pub/wiscombe/Multiple_Scatt/DISORT2.0beta

(note: the directory and file names tend to change).  The relevant

Fortran files of the DISORT distribution are provided in the file

"disort2.f".  Although disortsh is written in Fortran 90 and

automatically allocates the appropriate array sizes, there are array

sizes internal to DISORT that may need to be changed.  In disort2.f look

for "LOCAL SYMBOLIC DIMENSIONS" and change MXCLY, MXULV, MXCMU, MXUMU,

and MXPHI as needed.  Note: there is a dramatic increase in running time

if these array sizes are much larger than needed for the particular

problem.  See DISORT.doc for description of the DISORT routine and

parameters.

DISORTSH reads the same optical property files as SHDOMPP.  Its user

inputs read from stdin are also similiar to those of SHDOMPP, and its

output file format is also similiar.  Run disortsh interactively, or

look at the scripts or the USER_INPUT subroutine to see the required

input parmeters.

PPMIEPRP

ppmieprp.f90 makes optical properties files for SHDOMPP and DISORTSH

using Mie table files output by cloudprp or make_mie_table (in the SHDOM

distribution). The integrated mass content (e.g. liquid water path) and

effective radius for each of several particle components are specified

for each layer.  Molecular Rayleigh scattering is included with a Rayleigh

scattering coefficient [K/(km mb)] that depends on wavelength.  The

pressure at the layer boundaries, from which the integrated air density

is calculated, are computed by the program from the input height and

temperature profile.  A molecular absorption for each layer is also

specified; these must be computed with another program such as MODTRAN.

Rayleigh scattering coefficient:  The molecular scattering coefficient

depends on the air density.  Hence for a particular solar wavelength

band a coefficient (k) may be defined which gives the extinction from

the air pressure (p) and temperature (T):  ext = k p/T.  The Rayleigh

scattering coefficient input may be derived from the molecular Rayleigh

scattering optical depth.  The Rayleigh optical depth at sea level as a

function of wavelength (lambda in micron) is

tau = 0.0088*lambda**(-4.15+0.2*lambda)

From this formula the cloudprp Rayleigh input coefficient is

k = (2.97E-4 K mb^-1 km^-1) lambda**(-4.15+0.2*lambda)

Parameter      Description

NCOMP        number of optical components (i.e. number of Mie table files)

MIEFILES(*)  names of the Mie table files made by cloudprp

NLAY         number of layers

HEIGHTS(*)   heights of layer boundaries from the top down (km)

TEMPS(*)     layer boundary temperatures from the top down (Kelvin)

For each layer:

LWP(*)       liquid water paths (g/m^2) of each component

REFF(*)      effective radius (micron) of each component

MOLABS(*)    molecular absorption optical depth for each layer

RAYLCOEF     molecular Rayleigh scattering coefficient (K/(km mb))

PROPFILE     name of output property file

DISTRIBUTION FILES

shdompp.f90              SHDOMPP code (all of it)

fftpack5.1cos.f90        FFT routines

shdompp.doc              this documentation file

disortsh.f90             DISORT driver program with SHDOMPP like interface

disort2.f                DISORT routines

DISORT.doc               DISORT documentation file

ppmieprp.f90             program to make SHDOMPP input property files

makefile                 make file to compile distribution programs

run_shdompp_examples     csh script of examples of running SHDOMPP/DISORTSH

run_compare_shdompp      csh script to run SHDOMPP/DISORTSH comparison

compare_shdompp_disort.txt   results of SHDOMPP/DISORTSH comparison

put.c                    utility program for the scripts

dust_w0.55.mie           dust aerosol Mie table for 0.55 um

seasalt_w0.86.mie        sea salt aerosol Mie table for 0.86 um

seasalt_w2.13.mie        sea salt aerosol Mie table for 2.13 um

water_w0.86.mie          cloud droplet Mie table for 0.86 um

water_w2.13.mie          cloud droplet Mie table for 2.13 um

water_w10.7.mie          cloud droplet Mie table for 10.7 um

swrrtm_mls.ckd           shortwave RRTM k-distribution file for MLS atmosphere

swrrtm_liq_8.mie         SW RRTM band 8 cloud droplet Mie table